POLE 1
#include <WiFi.h>

#include <HTTPClient.h>

#include <HardwareSerial.h>

// Pin definitions

#define CURRENT_SENSOR_PIN 34

#define VOLTAGE_SENSOR_PIN 35

#define RELAY_PIN 2

#define GSM_RX 16

#define GSM_TX 17

// Sensor calibration

const int ACS712_mVperAmp = 185; // For 5A version

const float VOLTAGE_CALIBRATION = 250.0;

HardwareSerial gsmSerial(2);

String poleID = "POLE_001"; // Unique ID for each pole

void setup() {

Serial.begin(115200);

gsmSerial.begin(9600, SERIAL_8N1, GSM_RX, GSM_TX);

pinMode(RELAY_PIN, OUTPUT);

digitalWrite(RELAY_PIN, LOW);

initializeGSM();

delay(2000);

}

void loop() {

float current = readCurrent();

float voltage = readVoltage();

// Create data packet

String dataPacket = poleID + "," + String(voltage, 2) + "," + String(current, 2);

// Send data to central station

sendDataToRaspberryPi(dataPacket);

// Check for relay control commands

checkForCommands();

delay(30000); // 30 second interval

}

float readCurrent() {

int sensorValue = analogRead(CURRENT_SENSOR_PIN);

float voltage = (sensorValue * 3.3) / 4095.0; // Convert to voltage

float current = ((voltage - 1.65) / 0.185); // ACS712 5A sensitivity

return abs(current);

}

float readVoltage() {

int rawValue = analogRead(VOLTAGE_SENSOR_PIN);

float sensorVoltage = (rawValue * 3.3) / 4095.0;

float acVoltage = sensorVoltage * VOLTAGE_CALIBRATION / 3.3;

return acVoltage;

}

void sendDataToRaspberryPi(String data) {

gsmSerial.println("AT+HTTPINIT");

delay(2000);

gsmSerial.println("AT+HTTPPARA=\"CID\",1");

delay(1000);

gsmSerial.println("AT+HTTPPARA=\"URL\",\"http://raspberry-pi-ip:5000/data\"");

delay(1000);

gsmSerial.println("AT+HTTPPARA=\"CONTENT\",\"application/json\"");

delay(1000);

String postData = "{\"pole_id\":\"" + poleID + "\",\"data\":\"" + data + "\"}";

gsmSerial.println("AT+HTTPDATA=" + String(postData.length()) + ",10000");

delay(500);

gsmSerial.println(postData);

delay(10000);

gsmSerial.println("AT+HTTPACTION=1");

delay(5000);

gsmSerial.println("AT+HTTPTERM");

}

void checkForCommands() {

gsmSerial.println("AT+CMGR=1"); // Read SMS

delay(2000);

String response = gsmSerial.readString();

if (response.indexOf("DISCONNECT") != -1) {

digitalWrite(RELAY_PIN, HIGH); // Activate relay

Serial.println("Line disconnected by central command");

}

}



POLE 2
#include <SoftwareSerial.h>

// Pin definitions

#define BT_RX 2

#define BT_TX 3

#define VOLTAGE_SWITCH 4

#define STATUS_LED 13

SoftwareSerial bluetooth(BT_RX, BT_TX);

int lastVoltageState = -1;

unsigned long lastTransmission = 0;

const unsigned long TRANSMISSION_INTERVAL = 2000; // 2 seconds

void setup() {

Serial.begin(9600);

bluetooth.begin(9600);

pinMode(VOLTAGE_SWITCH, INPUT_PULLUP);

pinMode(STATUS_LED, OUTPUT);

Serial.println("Pole 2 Slave Started");

Serial.println("Connecting to Pole 1...");

// Initial connection indicator

for(int i = 0; i < 5; i++) {

digitalWrite(STATUS_LED, HIGH);

delay(200);

digitalWrite(STATUS_LED, LOW);

delay(200);

}

delay(2000);

}

void loop() {

// Read voltage state (switch position)

int currentVoltageState = digitalRead(VOLTAGE_SWITCH);

// Send data every 2 seconds or when state changes

if ((millis() - lastTransmission > TRANSMISSION_INTERVAL) ||

(currentVoltageState != lastVoltageState)) {

sendVoltageStatus(currentVoltageState);

lastVoltageState = currentVoltageState;

lastTransmission = millis();

// Update status LED

if (currentVoltageState == HIGH) {  // Switch open = voltage present

digitalWrite(STATUS_LED, HIGH);

Serial.println("Voltage OK");

} else {  // Switch closed = no voltage (fault simulation)

digitalWrite(STATUS_LED, LOW);

Serial.println("Voltage FAULT");

}

}

delay(100);

}

void sendVoltageStatus(int voltageState) {

String message = "POLE2,VOLTAGE:";

if (voltageState == HIGH) {

message += "1";  // Voltage present

} else {

message += "0";  // No voltage (fault)

}

bluetooth.println(message);

Serial.println("Sent: " + message);

}